| JOB NUMBER: |  |  | {{JOB_NUMBER}} |  | CUSTOMER: |  | MANSFIELD DISTRICT COUNCIL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SITE ADDRESS: |  | {{SITE_ADDRESS}} |  |  |
| ERECT DATE: |  |  |  |  |  |  |  |  |  |
| ORDER NUMBER: |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | PHONE NO.: |  |  |  |  |
| JOB DESCRIPTION: |  |  |  |  |  |  |  |  |  |
| SITE INFORMATION: |  |  |  |  |  |  |  |  |  |
| JOB DETAIL: Please tick as appropriate and add any onsite additions / changes to the job. |  |  |  |  |  |  |  |  |  |
| ✓ | SCAFFOLD TYPE |  |  | SIZE |  | ✓ | SCAFFOLD TYPE |  | SIZE |
|  | FRONT |  |  |  |  |  | TRIPLE HANDRAIL |  |  |
|  | SIDE |  |  |  |  |  | GINWHEEL |  |  |
|  | REAR |  |  |  |  |  | BRICKGUARDS |  |  |
|  | STACK |  |  |  |  |  | PADDING |  |  |
|  | GABLE |  |  |  |  |  | HILTI TIES |  |  |
| ERECTED |  | GANG |  | 1ST INSPECTION& PHOTO |  | HANDOVER |  | OFF HIRED | DISMANTLED |
|  |  | Andrew & Stef |  |  |  |  |  |  |  |
| * * * CORRECT P.P.E. MUST BE WORN AT ALL TIMES ! * * * |  |  |  |  |  |  |  |  |  |
| * * DO NOT FORGET TO TAKE A PHOTO OF COMPLETED JOB BEFORE HANDOVER ! * * |  |  |  |  |  |  |  |  |  |

ToughScaff Scaffolding Risk Assessment

All operatives to sign acknowledgement of understanding of the contents of this Risk Assessment

| Undertaken by: | DAVID KNOWLES | Signature: DKnowles | Date: {{TODAY_DATE}} |
| --- | --- | --- | --- |

I have had the contents of the above Risk Assessment explained to me and I fully understand its contents and will adhere to it at all times.

| Print Name | Sign | Date |
| --- | --- | --- |
| ANDREW BEASLEY |  | {{TODAY_DATE}} |
| STEFEN WESTMORELAND |  | {{TODAY_DATE}} |

- When completing the risk assessment, please be aware what the definitions of Hazards and Risks are – “Hazard” Something with a potential to cause harm “Risk” The likelihood that harm from the hazard will take place

- For the Risk Assessment and Residual Risk Assessment, please use the terms High, Moderate, Low and Insignificant to reflect the level of risk.

- Where the Residual Risk Assessment is High or Moderate, the control measures identifies are inadequate and you will need to consider alternatives to reduce the risk assessment to Low of Insignificant.

- If in doubt please consult your Manager.

| CUSTOMER NAME: MANSFIELD DISTRICT COUNCIL SITE: SEE JOB SHEET FOR REFERENCE |  |  | JOB / ORDER NUMBER: SEE JOB SHEET FOR REFERENCE WORK AREA: SEE JOB SHEET FOR REFERENCE | PAGE 1 OF 2 |
| --- | --- | --- | --- | --- |
| Hazard | At Risk | Risk Assessment | Preventative/Control Measures/Persons | Residual Risk Assessment |
| Contact with delivery/collection vehicles. | Scaffolders, supervision, site personnel and the general public | High | Materials to be delivered/collected from site by lorry utilising an agreed traffic route. Banksman will be used where the lorry needs to reverse. | Low |
| Manual handling during unloading/loading and during movement of materials on site. | Scaffolders / general public | Medium | Materials to be unloaded as required, where materials are to be transported across a public footpath warning signs are to be used and public kept out of working area. Materials to be transported to the work area by manual handling. Materials to be raised or lowered by passing hand to hand or utilising a gin wheel and or rope. When handling materials good manual handling technique to be utilised. | Low |
| Abrasion, splinters from handling scaffold boards, lubricant from handling scaffold fittings. | Scaffolders | Medium | All site personnel to wear suitable gloves whilst handling scaffold materials. These should be good quality safety gloves offering high cut resistance (level 3 or equivalent), high durability and excellent dexterity. | Low |
| Trips and slips over our materials whilst being stored on site or whilst being used during erection/dismantle. | Scaffolders, supervision, site personnel, residents and the general public | Medium | Work area to be cordoned off by suitable means to prevent access to site operatives from other sub-contractors, residents/public. Storage of materials within the work area will be subject to quantities that can be used within work period. Where it is not possible to maintain cordoned off areas, materials to be stored on lorry until required. No materials will be left unsecured out of scaffolder’s work hours, all spare materials will be removed from the site at the end of each shift. Customer to ensure general access and egress on site remains suitable and that residents are kept away from work activity areas as is reasonably practicable. | Low |
| Fall of materials during movement on site and during erection / dismantle. | Scaffolders, supervision, site personnel, residents and the general public | High | The work area is to be cordoned off using suitable means to prevent access. Materials to be stored by suitable means to prevent inadvertent movement. Personnel to wear helmets and safety boots. No materials will be left unattended at any time. Spare materials will be removed from site at the end of each shift. Crash decks will be provided over entrances as required. Right of way to be given to residents at all times. | Low |
| Fall of persons during erection / dismantle. | Scaffolders and supervision | High | Scaffolders to wear a suitable full body harness/lanyard and work from a fully-boarded properly supported working platform. The harness to be used in accordance with current NASC Guidance Note SG4:15 Preventing Falls in Scaffolding. Scaffolds will be erected methodically and dismantled in accordance with industry best practice (as per below) and utilising the “Scaffstep” advanced guardrail instillation platform (where applicable) for collective fall protection. | Low |
| Hazard | At Risk | Risk Assessment | Preventative/Control Measures/Persons | Residual Risk Assessment |
| Collapse of scaffold during erection / dismantle. | Scaffolders, supervision, site personnel, residents and the general public | High | Scaffold to be erected in accordance with TG20:21 / BSEN 12811. The scaffold will be erected and dismantled progressively. Hook and ring anchors used for tying, will be erected/dismantled progressively with the scaffold. No spare materials will be stored on the scaffold unless it is being used or removed before/after end of each shift. If any drilled anchors are used for ties these will be tested in accordance with NASC TG4:19. | Low |
| Access to scaffold during erection, prior to formal inspection, handover and during dismantle. | Site personnel, residents and general public | High | On commencement of erection and dismantle an “incomplete” scaffold sign to be displayed adjacent to the access routes. Physical barriers to be erected to prevent access to the immediate work area, where reasonably practicable. | Low |
| Collapse of scaffold when erected. | Site personnel, residents and general public | High | Customer to use scaffold in accordance with the loading specified on the handover certificate. No modifications to be made to scaffold. Scaffold to be inspected before first use, weekly and a record maintained by ToughScaff Limited. | Low |
| Contact with scaffolding when erected. | Scaffolders, supervision, site personnel, residents and the general public | High | Where scaffolds are erected on access routes, the 1st lift is to be erected at a height that allows free access under the scaffold. In circumstances where this is not reasonably practicable, the scaffold is to be handrailled-off using suitable means to prevent access. The standards of any access routes will be padded to prevent injuries on impact and will have high visibility markings. ToughScaff will endeavour to cordon off current work areas and keep others out, however it is the customer’s responsibility to ensure others cannot gain access to the scaffold when the scaffold is in use. | Low |
| Environmental conditions (wind, rain, etc.). | Scaffolders and supervision | Medium | Supervision will monitor the environmental working conditions to ensure that employees are not put at risk. Wet weather gear may be utilised, however Supervision will determine when work will cease due to environmental conditions and the Work at Height Regulations 2005. | Low |
| Electricity. | Scaffolders and supervision | High | Prior to any works starting, Customer to identify electrical hazard and be responsible for any isolation work that may be required. No works to commence until any present electrical hazards are isolated by qualified personnel. | Low |
| Substances hazardous to health ASBESTOS. | Scaffolders, supervision, site personnel, residents and the general public | High | Customer has advised ToughScaff of potential asbestos risk at scaffold request stage. ToughScaff operatives have Asbestos awareness training – certificates already in place from Safety & Access Limited. | Low |
| Competence, Supervision and Information. | Employees, Supervisors and Managers | High | Scaffolders/Supervisors will be trained to CISRS standards at the level of scaffolding that they erect. Scaffold Inspectors to have scaffold inspection training. All ToughScaff operatives to have received training on SG4:15 and “Scaffstep”. Refer to scaffolder’s CISRS cards. | Low |

| Method Statement / Scaffold Plan Erection and Dismantle of Independent-tied Scaffold to 1/2/3/4-Storey Property FOR MANSFIELD DISTRICT COUNCIL |  |
| --- | --- |
| CUSTOMER: Mansfield District Council | NAME OF PERSON COMPLETING FORM: David Knowles |
| CONTRACT: Scaffolding for Day-to-Day Maintenance & Repairs | DATE COMPLETED: |
| ACTIVITY: Erect & Dismantle of Independent-tied Scaffold to 1/2/3/4-Storey Property |  |

| SCOPE OF WORK Supply, erect and dismantle a general-purpose tube and fitting independent-tied scaffold to 1/2/3/4-storey property, erected to 5 boards wide, (with inside boards if necessary). All this in accordance with BS EN 12811 and NASC TG20:21. Scaffold to be erected to max bay lengths of 2m and max lift heights of 2.7m first lift then 2m lifts thereafter. |  |  |  |
| --- | --- | --- | --- |
| PROGRAMME START DATE: | DURATION: 1-day (guidance only) |  | WORKING DAYS: Mon – Fri WORKING HOURS: 08:00 – 16:30 |
| SUPERVISION Foreman: Andrew Beasley Emergency / Supervisor: David Knowles |  | Contact No.: 07967 519715 Contact No.: 07825 533 527 |  |
| PERSONNEL All CISRS Scaffolders engaged in this work activity will contain a balance of trained and experienced scaffolding operatives in accordance with the type of scaffolding required. Scaffold inspectors will have scaffold inspection training. Operatives already have asbestos awareness training. |  |  |  |
| EQUIPMENT TO BE USED IN WORK ACTIVITY Scaffolding tubing and couplers to BSEN 39: 2001 BS 1139. Scaffold couplers to BS1139/EN74. Scaffold boards to BS2482 or A grade. Ladders to BS1129 or BSEN131. |  |  |  |
| PPE REQUIREMENTS Standard Equipment is: Safety Footwear, Safety Helmet, Safety Harnesses/lanyard, High Visibility Vests and Gloves (level 3 cut resistance) in line with British Standards. Other items of PPE will be determined by risk assessment or by client requirement. |  |  |  |
| WORK AREAS / STORAGE / LOADING / UNLOADING / MOVEMENT OF MATERIALS Work areas will be segregated by means of physical barriers with signs designating this, where reasonable. Materials will be stored at a minimum, kept in good order and spare materials removed after works are completed. Materials will be loaded / unloaded by manual handling. Materials will be moved during scaffolding operations by either: Hand-to-hand or Gin wheel and rope. Spare materials will be removed from site at the end of each shift. EMERGENCY PROCEDURES TougScaff operatives will comply with customer site emergency procedures. The main contractor will provide onsite first-aid facilities for all ToughScaff operatives. All ToughScaff operatives will report any accidents or near misses to site supervisor as well as ToughScaff immediately and cooperate in the investigation. Incidents to be recorded in the Accident Book. |  |  |  |
| ENVIRONMENTAL IMPACT All scaffolding materials will be removed from site upon completion of works. Any substances that are subject to COSHH will have appropriate COSHH assessments and will only be used onsite with the customer’s permission. For example, Scaffeze. |  |  |  |
| NOTES FOR SCAFFOLD USERS Scaffold users are directly responsible for ensuring that the scaffold is ONLY USED for its intended purpose and loadings. Users must ensure that unauthorised persons do not interfere with the scaffold in any manner. Any modifications must be carried out by ToughScaff’s trained and competent Scaffolders. Users must ensure that the scaffold is inspected before 1st use, after substantial alterations, after any event that may affect its stability, bad weather and every 7 days. Any queries regarding this structure to be raised with ToughScaff: 01623 753899. |  |  |  |

WORK METHOD / PROCEDURE Erect tube and fitting Independent scaffolding in accordance with BSEN12811 and NASC TG20:21 and site requirement using trained and competent CISRS Scaffolders working in accordance with NASC guidance note SG4:15 `Preventing falls in scaffolding Operations’ as follows: Base and 1st lift Level - Base scaffold out on the ground prepared by the Customer. - Erect standards (on top of baseboards and baseplates) and ledgers for 1st lift. Ledgers will be placed well above head height at all doorways and essential access routes. - Fix 1st lift transoms and erect single guardrails to the level above using “Scaffstep” temporary work platform or “Fast guard” push pole system. - Fully board scaffold from ground level. - Fix permanent ladder access at point agreed with client using same to gain access to next lift of scaffold. Signs will be attached warning that scaffold is incomplete. - Scaffolders to access 1st lift only when fully boarded and protected by a single guardrail. - Base standards at access ways will be designated with hi-visibility markings. - All access doorways will be protected by crash decking (single layer of scaffold boards) over same at first lift height. 2nd and Subsequent Lifts - Fix 2nd lift ledgers, braces and transoms from boarded scaffold platform. - Stabilize scaffold using buttresses/rakers/drilled anchor ties (whichever appropriate). - Raise scaffold boards to 2nd lift from the 1st lift (Scaffolders will hook-on at this point as there are 4m anchor points available and to prevent ops falling into gaps created). - Single guardrails will be fitted to new working lift prior to erecting subsequent lifts using the “Scaffstep” or “Fast guard”. - These procedures will be repeated on any subsequent lifts and any table lifts. - Double guardrails, toeboards and inside boards along with brickguards will be placed onto top lift - All completed ladders will be fitted with inward opening self-closing safety gates. - Scaffold will be inspected by ToughScaff and then handed over to Client upon satisfactory completion. Dismantle Scaffold - Scaffold will be inspected prior to dismantle commencing to ensure that ties and bracing are in place and that no debris is on scaffold. - Any barriers required will be erected around work area and scaffold incomplete do not use signs placed at access points. - Working in reverse of the erection process Scaffolders will dismantle progressively (normally in reverse of the erection procedure) working back towards the safety of the ladder access at all times, ensuring that the Scaffolders have their harnesses clipped on when not in the “Scaffolders Safe Zone” as detailed in SG4:15 - Buttressing/Rakers and bracing will be removed progressively in line with the dismantle procedure. - Scaffolders to ensure that scaffold does not get overloaded with dismantled materials and that the scaffold remains stable during all dismantle activities. No spare dismantled materials will be left on site at end of shift. - All materials will be passed hand to hand or by rope and wheel. Under no circumstances will any materials be BOMBED!! - All materials will be cleared from site at end of each shift!

| MONITORING AND COMPLIANCE Supervision will ensure that this Method Statement is adhered to by means of regular monitoring. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WORK METHODS / PROCEDURES I confirm that I have been instructed on the method of work, the hazards and risks involved as well as the necessary control measures. |  |  |  |  |  |  |  |  |  |  |  |  |
| NAME |  | SIGNATURE |  | DATE |  | NAME |  |  | SIGNATURE |  | DATE |  |
| ANDREW BEASLEY |  |  |  | {{TODAY_DATE}} |  |  |  |  |  |  |  |  |
| STEFEN WESTMORELAND |  |  |  | {{TODAY_DATE}} |  |  |  |  |  |  |  |  |
| PROCEDURES FOR CHANGING THIS METHOD STATEMENT If necessary this Method statement can be changed by immediate Supervisor/Partner only. |  |  |  |  |  |  |  |  |  |  |  |  |
| ToughScaff Quotation Reference No.: |  |  |  |  |  |  |  |  |  |  |  |  |
| TG20:21 Compliance Sheet /Scaffold Design Reference No.: |  |  |  |  |  |  |  |  |  |  |  |  |
| Customer / Other Design Drawing Reference No.: |  |  |  |  |  |  |  |  |  |  |  |  |
| HANDOVER PROCEDURE Following completion of the scaffolding works, the scaffold inspector will carry out a visual inspection of the structure accompanied (ideally) by a representative of the customer. Upon satisfactory completion of this ToughScaff will issue a handover certificate. |  |  |  |  |  |  |  |  |  |  |  |  |
| INSPECTION PROCEDURE If contracted to do so, ToughScaff will carry out statutory inspections in accordance with the Work at Height Regulations 2005 and record its findings accordingly. |  |  |  |  |  |  |  |  |  |  |  |  |
| Review Period |  |  | Review / inspection period to be executed in line with requirements of statutory inspections Yes / No If ‘No’, state specific site requirements |  |  |  |  |  |  |  |  |  |
| Distribution* | Partner |  | Designer | Contractor | Customer |  | Safety Consultant |  | Supervisor | Operatives |  | Job File |

* Delete as necessary